# EcoSynTech Firmware V8.5.0

Phiên bản: 8.5.0 | Ngày: 10.04.2026

ESP32 IoT Firmware cho nông nghiệp thông minh

# 1. Tính năng chính

• Watchdog 30 giây - Tự động reset khi treo

• Auto-recovery - Restart khi health score < 30

• Device Health Score - Tính điểm sức khỏe (0-100)

• WiFi/MQTT với reconnect + backoff

• Deep sleep thông minh

• Lưu backlog khi offline

• OTA update

• HMAC-SHA256 security + Nonce

# 2. Auto-recovery Logic

Tự động restart khi:

• Health score < 30

• WiFi fail >= 10 lần liên tiếp

• MQTT fail >= 10 lần liên tiếp

• Heap < 30KB

# 3. Health Score Calculation

• Base: 100 điểm

• WiFi fail * 5: -5 điểm/fail

• MQTT fail * 5: -5 điểm/fail

• Heap < 20KB: -20 điểm

• Heap < 30KB: -10 điểm

• Backlog > 100: -15 điểm

# 4. Pinout (EcoSynTech 5.3 PRO-X)

• SD Card: CS=5, SCK=18, MOSI=23, MISO=19

• I2C: SDA=21, SCL=22

• DHT22: GPIO 33

• DS18B20: GPIO 32

• Soil ADC: GPIO 36

• Battery ADC: GPIO 35

• Relays: 26, 27, 25, 14

# 5. Webhook URL

Default: https://script.google.com/macros/s/AKfycbypoJdDXqnBAWwkuQGRZvaNG0OigGlfA_STVaYLA0cSMxkCQK5WMF2WbuyEDa_0xCbrQg/exec

# 6. MQTT Configuration

• Broker: mqtt.ecosyntech.com

• Port: 8883 (TLS)

• Topics: ecosyntech/{device_id}/sensor, command, heartbeat, status

# 7. Command Reference

• relay1_on, relay1_off

• relay2_on, relay2_off

• relay3_on, relay3_off

• relay4_on, relay4_off

• reboot

• set_deep_sleep

• maintenance_on/off

# 8. Build & Upload

Sử dụng Arduino IDE với board ESP32

Library: ArduinoJson, PubSubClient, WiFiManager, DallasTemperature, DHT, Adafruit_ADS1X15