Dưới đây là phiên bản firmware đã được tối ưu và bổ sung Watchdog phần cứng, tự động phục hồi, quản lý năng lượng thông minh cùng các tính năng đặc thù cho nông nghiệp thông minh:

· ✅ Watchdog (ESP32 Task Watchdog Timer) – tự động reset nếu vòng lặp chính bị treo.

· ✅ Giám sát kết nối WiFi & MQTT với cơ chế backoff và reset sau 10 lần thất bại liên tiếp.

· ✅ Chế độ deep sleep thông minh – chỉ sleep khi không có lệnh chờ, lưu trạng thái trước khi sleep.

· ✅ Cảnh báo nông nghiệp: nhiệt độ cao, độ ẩm thấp, pin yếu, mất kết nối kéo dài.

· ✅ Tự động điều khiển thiết bị dựa trên cảm biến (tưới, quạt, đèn) với hysteresis và cooldown.

· ✅ Lưu trữ sự kiện chi tiết (SD/SPIFFS) và gửi lên cloud khi có kết nối.

· ✅ Xử lý lỗi SD/SPIFFS – fallback lẫn nhau, tự động tạo thư mục.

· ✅ Tối ưu bộ nhớ – giảm sử dụng RAM động, dùng std::vector có giới hạn, xóa nonce cũ.

· ✅ Hỗ trợ OTA với kiểm tra watchdog trước khi reboot.

· ✅ Thêm cảm biến ánh sáng (ví dụ BH1750 – có thể mở rộng) và cảnh báo thiếu sáng.

/*

EcoSynTech Firmware V8.5.0 PRO-X SECURE + MQTT + WATCHDOG

- Thêm watchdog, tự động phục hồi, tối ưu nông nghiệp

- Bảo mật HTTPS + MQTT với nonce, HMAC, replay protection

- Quản lý năng lượng thông minh (deep sleep khi không hoạt động)

- Tương thích ESP32, hỗ trợ SD, SPIFFS, cảm biến DS18B20, DHT22, ADS1115, độ ẩm đất, pin

*/

#include <Arduino.h>

#include <WiFi.h>

#include <WiFiClientSecure.h>

#include <HTTPClient.h>

#include <PubSubClient.h>

#include <SPI.h>

#include <SD.h>

#include <SPIFFS.h>

#include <Preferences.h>

#include <ArduinoJson.h>

#include <WiFiManager.h>

#include <time.h>

#include <mbedtls/md.h>

#include <mbedtls/sha256.h>

#include <Update.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#include <DHT.h>

#include <Wire.h>

#include <Adafruit_ADS1X15.h>

#include <esp_task_wdt.h>

#include <vector>

#include <map>

#include <algorithm>

#include <math.h>

// ============================ Version & Watchdog ============================

#define FW_VERSION "8.5.0"

#define WATCHDOG_TIMEOUT_SEC 30          // 30 giây watchdog

#define MAX_CONSECUTIVE_FAILS 10         // số lần fail liên tiếp để reboot

#define TZ_OFFSET_SECONDS (7 * 3600)

// ============================ Defaults ============================

#define DEFAULT_HMAC_SECRET " CEOTAQUANGTHUAN_TADUYANH_CTYTNHHDUYANH_ECOSYNTECH_2026"

#define DEFAULT_API_KEY " TADUYANH09082012_ECOSYNTECH_2026"

#define DEFAULT_WEBHOOK_URL " https://script.google.com/macros/s/AKfycbypoJdDXqnBAWwkuQGRZvaNG0OigGlfA_STVaYLA0cSMxkCQK5WMF2WbuyEDa_0xCbrQg/exec "

#define DEFAULT_MQTT_BROKER "mqtt.ecosyntech.com"

#define DEFAULT_MQTT_PORT 8883

#define DEFAULT_MQTT_USER "ecosyntech"

#define DEFAULT_MQTT_PASS "change_me"

// ============================ Pinout (EcoSynTech 5.3 PRO-X) ============================

#define SD_CS_PIN 5

#define PIN_SD_SCK 18

#define PIN_SD_MOSI 23

#define PIN_SD_MISO 19

#define PIN_SDA 21

#define PIN_SCL 22

#define ADS1115_ADDR 0x48

#define PIN_DHT22_DATA 33

#define PIN_DS18B20_DATA 32

#define PIN_SOIL_ADC 36

#define PIN_BATTERY_ADC 35

#define RELAY_PIN1 26

#define RELAY_PIN2 27

#define RELAY_PIN3 25

#define RELAY_PIN4 14

#define RELAY_ACTIVE_LEVEL HIGH

#define RELAY_INACTIVE_LEVEL LOW

// ============================ Timing ============================

#define SENSOR_INTERVAL_MS (60UL * 1000UL)

#define HEARTBEAT_INTERVAL_MS (60UL * 60UL * 1000UL)

#define BATCH_SYNC_INTERVAL_MS (6UL * 60UL * 60UL * 1000UL)

#define BACKLOG_RETRY_INTERVAL_MS (2UL * 60UL * 1000UL)

#define OTA_CHECK_INTERVAL_MS (12UL * 60UL * 60UL * 1000UL)

#define CLEANUP_INTERVAL_MS (24UL * 60UL * 60UL * 1000UL)

#define COMMAND_CHECK_INTERVAL_MS (10UL * 1000UL)

#define WIFI_RECONNECT_MIN_MS (30UL * 1000UL)

#define RULE_COOLDOWN_MS (5000UL)

#define MAX_HANDLED_COMMANDS 100

#define MAX_EVENT_LOG_SIZE 8192

#define MAX_BACKLOG_FILES_SPIFFS 2000

#define MAX_BACKLOG_BYTES_SD (50UL * 1024UL * 1024UL)

#define COMMAND_TTL_MS (7UL * 24UL * 60UL * 60UL * 1000UL)

#define REPLAY_WINDOW_SEC 1200UL

#define NONCE_EXPIRY_SEC 1200UL

#define RATE_LIMIT_PER_DEVICE 20UL

#define LOW_BATTERY_THRESHOLD_V 3.5f

#define CRITICAL_BATTERY_THRESHOLD_V 3.2f

#define TEMP_ANOMALY_THRESHOLD_C 40.0f

#define SOIL_DRY_THRESHOLD_PERCENT 30.0f

#define INVALID_SENSOR_VALUE (-999.0f)

#define DEFAULT_DEEP_SLEEP_SEC 3600UL

#define MIN_INTERVAL_SEC 60UL

static constexpr float BATTERY_DIVIDER_RATIO = (100000.0f + 33000.0f) / 33000.0f;

// ============================ Files ============================

#define BACKLOG_DIR "/backlog"

#define ARCHIVE_DIR "/archive"

// ============================ Cấu trúc dữ liệu ============================

struct DynamicConfig {

uint32_t post_interval_sec = 3600;

uint32_t sensor_interval_sec = 60;

bool deep_sleep_enabled = false;

bool ml_anomaly_enabled = false;

uint32_t max_data_age_days = 180;

uint32_t batch_sync_interval_sec = 21600;

uint32_t ota_check_interval_sec = 43200;

uint32_t cleanup_interval_sec = 86400;

uint32_t config_version = 0;

bool mqtt_enabled = true;

char mqtt_broker[128] = DEFAULT_MQTT_BROKER;

uint16_t mqtt_port = DEFAULT_MQTT_PORT;

char mqtt_user[64] = DEFAULT_MQTT_USER;

char mqtt_pass[64] = DEFAULT_MQTT_PASS;

bool mqtt_tls = true;

};

struct Batch {

String batch_id;

String type;

String start_date;

String end_date;

String status;

String zone;

bool force_send = false;

};

struct SensorReading {

String sensor_type;

float value = NAN;

String unit;

String sensor_id;

String zone;

String local_time;

String event_ts;

};

struct ControlRule {

String zone;

String sensor;

float minVal = 0;

float maxVal = 0;

float hysteresis = 0;

unsigned long durationSec = 0;

String action;

};

struct RuleState {

unsigned long triggeredAtMs = 0;

bool active = false;

float lastValue = NAN;

unsigned long lastActionMs = 0;

};

struct ConfigStatic {

char deviceId[48] = "ECOSYNTECH0001";

char deviceName[48] = "EcoSynTech_PROX";

char webhookUrl[256] = DEFAULT_WEBHOOK_URL;

char getBatchUrl[256] = DEFAULT_WEBHOOK_URL;

char otaUrl[256] = DEFAULT_WEBHOOK_URL;

char hmacSecret[128] = DEFAULT_HMAC_SECRET;

char apiKey[80] = DEFAULT_API_KEY;

bool sdEnabled = true;

bool deepSleepEnabled = false;

uint32_t deepSleepSec = DEFAULT_DEEP_SLEEP_SEC;

uint32_t pushIntervalSec = 3600;

} CFG;

enum class AppState { BOOT, WIFI_SETUP, TIME_SYNC, LOAD_CONFIG, RUNNING, DEGRADED_OFFLINE, OTA_PENDING, MAINTENANCE };

struct AppMetrics {

uint32_t sensorReadCount = 0;

uint32_t heartbeatCount = 0;

uint32_t postSuccessCount = 0;

uint32_t postFailCount = 0;

uint32_t commandExecCount = 0;

uint32_t otaSuccessCount = 0;

uint32_t otaFailCount = 0;

uint32_t wifiReconnectCount = 0;

uint32_t backlogSendCount = 0;

uint32_t backlogFailCount = 0;

uint32_t lastKnownFreeHeap = 0;

uint32_t mqttPublishCount = 0;

uint32_t mqttReconnectCount = 0;

uint32_t consecutiveWifiFails = 0;   // Thêm đếm fail liên tiếp

uint32_t consecutiveMqttFails = 0;

};

struct NonceRecord {

String nonce;

uint32_t timestamp;

};

// ============================ Globals ============================

Preferences prefs;

DynamicConfig DYN_CFG;

AppState APP_STATE = AppState::BOOT;

AppMetrics METRICS;

OneWire oneWire(PIN_DS18B20_DATA);

DallasTemperature ds18b20(&oneWire);

DHT dht(PIN_DHT22_DATA, DHT22);

Adafruit_ADS1115 ads(ADS1115_ADDR);

WiFiServer debugServer(80);

bool sdReady = false;

bool maintenanceMode = false;

volatile bool otaPending = false;

bool wifiEverConnected = false;

bool adsReady = false;

unsigned long lastSensorMs = 0, lastHeartbeatMs = 0, lastBatchSyncMs = 0, lastBacklogRetryMs = 0;

unsigned long lastOTAcheckMs = 0, lastCleanupMs = 0, lastConfigFetchMs = 0, lastCommandCheckMs = 0;

unsigned long lastWiFiReconnectAttemptMs = 0, lastNonceCleanupMs = 0, lastActivityMs = 0, bootMs = 0;

std::vector<Batch> activeBatches;

std::vector<ControlRule> controlRules;

std::map<String, RuleState> ruleStates;

std::vector<String> handledCommands;

String LAST_SENSOR_PAYLOAD = "";

uint32_t LAST_SENSOR_PAYLOAD_AT = 0;

bool LAST_SENSOR_PAYLOAD_VALID = false;

unsigned long outboundWindowStartMs = 0;

uint32_t outboundWindowCount = 0;

uint32_t eventLogWriteCount = 0;

std::vector<NonceRecord> nonceList;

const size_t MAX_NONCES = 100;

// MQTT

WiFiClientSecure mqttSecureClient;

WiFiClient mqttPlainClient;

PubSubClient mqttClient;

unsigned long lastMqttReconnectAttempt = 0;

bool mqttConnected = false;

// ============================ Forward declarations ============================

bool allowOutboundRequest();

bool extractJsonTimestamp(JsonVariantConst v, uint32_t& tsOut);

bool isWithinReplayWindow(JsonVariantConst v);

bool isCommandExpired(JsonObjectConst cmd);

void logEvent(const String& entityId, const String& eventType, const String& payload, const String& source);

void reportCommandResult(const String& commandId, const String& commandName, const String& status, const String& note);

void captureMediaStub();

bool postToBlockchain(const String& snapshotPayload, const String& deviceId);

void cleanupArchiveData();

String serializeJsonWithoutSig(JsonDocument &doc);

bool sendWrappedJSON(const String &rawPayloadJson);

bool fetchWithRetry(const String& url, String& payload, int maxRetries, uint32_t timeoutMs = 10000);

bool isNonceValid(const String& deviceId, const String& nonce, uint32_t ts);

void rememberNonce(const String& nonce, uint32_t ts);

void cleanupNonces();

void rememberCommandId(const String& commandId);

bool wasCommandHandled(const String& commandId);

void mqttCallback(char* topic, byte* payload, unsigned int length);

void mqttPublishSensorData(const String& payload);

void mqttPublishHeartbeat();

void mqttSubscribeCommands();

void mqttReconnect();

void processMqtt();

void executeLocalCommand(const String& command, JsonVariant params, const String& commandId);

bool checkOTA();

void enterDeepSleepIfNeeded();

// ============================ Tiện ích ============================

static inline bool wifiConnected() { return WiFi.status() == WL_CONNECTED; }

static inline bool isTimeValid() { return time(nullptr) >= 1600000000; }

void touchActivity() { lastActivityMs = millis(); }

void copyStringSafe(char* dst, size_t dstSize, const String& src) {

if (!dst || dstSize == 0) return;

snprintf(dst, dstSize, "%s", src.c_str());

}

String bytesToHex(const unsigned char *buf, size_t len) {

static const char hex[] = "0123456789abcdef";

String s; s.reserve(len * 2);

for (size_t i = 0; i < len; i++) {

s += hex[(buf[i] >> 4) & 0x0F];

s += hex[buf[i] & 0x0F];

}

return s;

}

String generateNonce() {

char nonce[33];

for (int i = 0; i < 16; i++) {

uint8_t b = (uint8_t)(esp_random() & 0xFF);

snprintf(nonce + i * 2, sizeof(nonce) - (i * 2), "%02x", b);

}

nonce[32] = '\0';

return String(nonce);

}

String genPayloadId() {

uint32_t a = (uint32_t)esp_random();

uint32_t b = millis();

char buf[40];

snprintf(buf, sizeof(buf), "%08X%08X", a, b);

return String(buf);

}

String iso8601UTC(time_t t) {

struct tm tm;

gmtime_r(&t, &tm);

char buf[32];

strftime(buf, sizeof(buf), "%Y-%m-%dT%H:%M:%SZ", &tm);

return String(buf);

}

String iso8601Local(time_t t) {

time_t localt = t + TZ_OFFSET_SECONDS;

struct tm tm;

gmtime_r(&localt, &tm);

char buf[32];

strftime(buf, sizeof(buf), "%Y-%m-%dT%H:%M:%S+0700", &tm);

return String(buf);

}

String getCurrentUTCTimestamp() { return iso8601UTC(time(nullptr)); }

String maskSecret(const String& s) {

if (s.length() <= 8) return s;

return s.substring(0,4) + "..." + s.substring(s.length()-4);

}

String computeSha256(const String& input) {

uint8_t hash[32];

mbedtls_sha256_context ctx;

mbedtls_sha256_init(&ctx);

mbedtls_sha256_starts_ret(&ctx, 0);

mbedtls_sha256_update_ret(&ctx, (const uint8_t*)input.c_str(), input.length());

mbedtls_sha256_finish_ret(&ctx, hash);

mbedtls_sha256_free(&ctx);

return bytesToHex(hash, 32);

}

String computeHmacSha256(const String& message, const String& key) {

const mbedtls_md_info_t* md_info = mbedtls_md_info_from_type(MBEDTLS_MD_SHA256);

if (!md_info) return "";

unsigned char out[32];

mbedtls_md_context_t ctx;

mbedtls_md_init(&ctx);

if (mbedtls_md_setup(&ctx, md_info, 1) != 0) {

mbedtls_md_free(&ctx);

return "";

}

bool ok = (mbedtls_md_hmac_starts(&ctx, (const unsigned char*)key.c_str(), key.length()) == 0) &&

(mbedtls_md_hmac_update(&ctx, (const unsigned char*)message.c_str(), message.length()) == 0) &&

(mbedtls_md_hmac_finish(&ctx, out) == 0);

mbedtls_md_free(&ctx);

if (!ok) return "";

return bytesToHex(out, 32);

}

String canonicalStringify(JsonVariantConst v) {

if (v.isNull()) return "null";

if (v.is<bool>()) return v.as<bool>() ? "true" : "false";

if (v.is<int>() || v.is<long>() || v.is<unsigned int>() || v.is<unsigned long>() || v.is<float>() || v.is<double>()) {

double n = v.as<double>();

if (isnan(n) || isinf(n)) return "null";

String s = String(n, 6);

while (s.indexOf('.') >= 0 && (s.endsWith("0") || s.endsWith("."))) {

if (s.endsWith(".")) { s.remove(s.length()-1); break; }

s.remove(s.length()-1);

}

return s;

}

if (v.is<const char*>() || v.is<String>()) {

String s = v.as<String>();

s.replace("\\", "\\\\");

s.replace("\"", "\\\"");

s.replace("\n", "\\n");

s.replace("\r", "\\r");

s.replace("\t", "\\t");

return "\"" + s + "\"";

}

if (v.is<JsonArrayConst>()) {

String out = "[";

JsonArrayConst arr = v.as<JsonArrayConst>();

for (size_t i = 0; i < arr.size(); i++) {

if (i) out += ",";

out += canonicalStringify(arr[i]);

}

out += "]";

return out;

}

if (v.is<JsonObjectConst>()) {

JsonObjectConst obj = v.as<JsonObjectConst>();

std::vector<String> keys;

for (JsonPairConst kv : obj) keys.push_back(String(kv.key().c_str()));

std::sort(keys.begin(), keys.end());

String out = "{";

for (size_t i = 0; i < keys.size(); i++) {

if (i) out += ",";

out += "\"" + keys[i] + "\":";

out += canonicalStringify(obj[keys[i].c_str()]);

}

out += "}";

return out;

}

return "null";

}

String joinPath(const String& dir, const String& name) {

if (name.startsWith("/")) return name;

if (dir.endsWith("/")) return dir + name;

return dir + "/" + name;

}

String currentBatchId() {

if (activeBatches.empty()) return "default_batch";

for (const auto& b : activeBatches)

if (b.force_send || String(b.status).equalsIgnoreCase("active"))

return b.batch_id;

return activeBatches[0].batch_id.length() ? activeBatches[0].batch_id : "default_batch";

}

String currentZone() {

if (activeBatches.empty()) return "default";

for (const auto& b : activeBatches)

if (b.force_send || String(b.status).equalsIgnoreCase("active"))

return b.zone.length() ? b.zone : "default";

return activeBatches[0].zone.length() ? activeBatches[0].zone : "default";

}

// ============================ Cấu hình & lưu trữ ============================

void normalizeConfig() {

if (CFG.deepSleepSec < MIN_INTERVAL_SEC) CFG.deepSleepSec = MIN_INTERVAL_SEC;

if (CFG.pushIntervalSec < MIN_INTERVAL_SEC) CFG.pushIntervalSec = MIN_INTERVAL_SEC;

if (DYN_CFG.post_interval_sec < MIN_INTERVAL_SEC) DYN_CFG.post_interval_sec = CFG.pushIntervalSec;

if (DYN_CFG.sensor_interval_sec < MIN_INTERVAL_SEC) DYN_CFG.sensor_interval_sec = 60;

if (DYN_CFG.batch_sync_interval_sec < MIN_INTERVAL_SEC) DYN_CFG.batch_sync_interval_sec = 21600;

if (DYN_CFG.ota_check_interval_sec < MIN_INTERVAL_SEC) DYN_CFG.ota_check_interval_sec = 43200;

if (DYN_CFG.cleanup_interval_sec < MIN_INTERVAL_SEC) DYN_CFG.cleanup_interval_sec = 86400;

if (DYN_CFG.max_data_age_days == 0) DYN_CFG.max_data_age_days = 180;

}

void saveConfig() {

normalizeConfig();

prefs.begin("ecofarm", false);

prefs.putBytes("config", &CFG, sizeof(CFG));

prefs.putUInt("post_int", DYN_CFG.post_interval_sec);

prefs.putUInt("sens_int", DYN_CFG.sensor_interval_sec);

prefs.putBool("deep_sl", DYN_CFG.deep_sleep_enabled);

prefs.putBool("ml_anom", DYN_CFG.ml_anomaly_enabled);

prefs.putUInt("max_age", DYN_CFG.max_data_age_days);

prefs.putUInt("batch_sync", DYN_CFG.batch_sync_interval_sec);

prefs.putUInt("ota_check", DYN_CFG.ota_check_interval_sec);

prefs.putUInt("cleanup_int", DYN_CFG.cleanup_interval_sec);

prefs.putUInt("cfg_ver", DYN_CFG.config_version);

prefs.putBool("mqtt_en", DYN_CFG.mqtt_enabled);

prefs.putString("mqtt_broker", String(DYN_CFG.mqtt_broker));

prefs.putUShort("mqtt_port", DYN_CFG.mqtt_port);

prefs.putString("mqtt_user", String(DYN_CFG.mqtt_user));

prefs.putString("mqtt_pass", String(DYN_CFG.mqtt_pass));

prefs.putBool("mqtt_tls", DYN_CFG.mqtt_tls);

prefs.end();

}

void loadStoredConfig() {

prefs.begin("ecofarm", true);

size_t sz = prefs.getBytes("config", &CFG, sizeof(CFG));

if (sz != sizeof(CFG)) Serial.println("[CFG] Using defaults for static config");

else Serial.println("[CFG] Loaded static config from NVS");

DYN_CFG.post_interval_sec = prefs.getUInt("post_int", CFG.pushIntervalSec);

DYN_CFG.sensor_interval_sec = prefs.getUInt("sens_int", 60);

DYN_CFG.deep_sleep_enabled = prefs.getBool("deep_sl", CFG.deepSleepEnabled);

DYN_CFG.ml_anomaly_enabled = prefs.getBool("ml_anom", false);

DYN_CFG.max_data_age_days = prefs.getUInt("max_age", 180);

DYN_CFG.batch_sync_interval_sec = prefs.getUInt("batch_sync", 21600);

DYN_CFG.ota_check_interval_sec = prefs.getUInt("ota_check", 43200);

DYN_CFG.cleanup_interval_sec = prefs.getUInt("cleanup_int", 86400);

DYN_CFG.config_version = prefs.getUInt("cfg_ver", 0);

DYN_CFG.mqtt_enabled = prefs.getBool("mqtt_en", true);

String broker = prefs.getString("mqtt_broker", DEFAULT_MQTT_BROKER);

strncpy(DYN_CFG.mqtt_broker, broker.c_str(), sizeof(DYN_CFG.mqtt_broker)-1);

DYN_CFG.mqtt_port = prefs.getUShort("mqtt_port", DEFAULT_MQTT_PORT);

String user = prefs.getString("mqtt_user", DEFAULT_MQTT_USER);

strncpy(DYN_CFG.mqtt_user, user.c_str(), sizeof(DYN_CFG.mqtt_user)-1);

String pass = prefs.getString("mqtt_pass", DEFAULT_MQTT_PASS);

strncpy(DYN_CFG.mqtt_pass, pass.c_str(), sizeof(DYN_CFG.mqtt_pass)-1);

DYN_CFG.mqtt_tls = prefs.getBool("mqtt_tls", true);

prefs.end();

normalizeConfig();

}

void saveHandledCommands() {

String joined;

for (size_t i = 0; i < handledCommands.size(); i++) {

if (i) joined += ",";

joined += handledCommands[i];

}

prefs.begin("ecofarm_cmd", false);

prefs.putString("handled_ids", joined);

prefs.end();

}

void loadHandledCommands() {

handledCommands.clear();

prefs.begin("ecofarm_cmd", true);

String raw = prefs.getString("handled_ids", "");

prefs.end();

if (raw.length() == 0) return;

int start = 0;

while (start < raw.length()) {

int sep = raw.indexOf(',', start);

if (sep < 0) sep = raw.length();

String id = raw.substring(start, sep);

id.trim();

if (id.length()) handledCommands.push_back(id);

start = sep + 1;

}

}

bool wasCommandHandled(const String& commandId) {

if (commandId.length() == 0) return true;

for (const auto& id : handledCommands) if (id == commandId) return true;

return false;

}

void rememberCommandId(const String& commandId) {

if (commandId.length() == 0) return;

if (wasCommandHandled(commandId)) return;

handledCommands.push_back(commandId);

if (handledCommands.size() > MAX_HANDLED_COMMANDS)

handledCommands.erase(handledCommands.begin());

saveHandledCommands();

}

// ============================ Nonce ============================

bool isNonceValid(const String& deviceId, const String& nonce, uint32_t ts) {

if (nonce.length() == 0 || ts == 0) return false;

uint32_t nowTs = (uint32_t)time(nullptr);

if (nowTs < ts || (nowTs - ts) > NONCE_EXPIRY_SEC) return false;

for (const auto& record : nonceList) if (record.nonce == nonce) return false;

if (wasCommandHandled(nonce)) return false;

return true;

}

void rememberNonce(const String& nonce, uint32_t ts) {

if (nonce.length() == 0) return;

uint32_t nowTs = (uint32_t)time(nullptr);

for (auto it = nonceList.begin(); it != nonceList.end(); ) {

if (nowTs - it->timestamp > NONCE_EXPIRY_SEC) it = nonceList.erase(it);

else ++it;

}

nonceList.push_back({nonce, nowTs});

if (nonceList.size() > MAX_NONCES) nonceList.erase(nonceList.begin());

}

void cleanupNonces() {

uint32_t nowTs = (uint32_t)time(nullptr);

for (auto it = nonceList.begin(); it != nonceList.end(); ) {

if (nowTs - it->timestamp > NONCE_EXPIRY_SEC) it = nonceList.erase(it);

else ++it;

}

}

// ============================ Lưu trữ (SD/SPIFFS) ============================

File openStorageFile(const String& path, bool writeMode) {

if (sdReady && CFG.sdEnabled) return SD.open(path, writeMode ? FILE_WRITE : FILE_READ);

return SPIFFS.open(path, writeMode ? FILE_WRITE : FILE_READ);

}

bool openStorageWrite(const String& path, const String& content) {

File f = openStorageFile(path, true);

if (!f) return false;

f.print(content);

f.flush();

f.close();

return true;

}

bool openStorageRemove(const String& path) {

if (sdReady && CFG.sdEnabled) return SD.remove(path);

return SPIFFS.remove(path);

}

uint32_t getStorageUsedBytes() {

uint32_t total = 0;

File dir = (sdReady && CFG.sdEnabled) ? SD.open(BACKLOG_DIR) : SPIFFS.open(BACKLOG_DIR);

if (!dir) return 0;

while (true) {

File entry = dir.openNextFile();

if (!entry) break;

if (!entry.isDirectory()) total += entry.size();

entry.close();

}

dir.close();

return total;

}

uint32_t getBacklogCount() {

uint32_t count = 0;

File dir = (sdReady && CFG.sdEnabled) ? SD.open(BACKLOG_DIR) : SPIFFS.open(BACKLOG_DIR);

if (!dir) return 0;

while (true) {

File entry = dir.openNextFile();

if (!entry) break;

if (!entry.isDirectory()) count++;

entry.close();

}

dir.close();

return count;

}

bool backlogHasRoom() {

if (sdReady && CFG.sdEnabled) return getStorageUsedBytes() < MAX_BACKLOG_BYTES_SD;

return getBacklogCount() < MAX_BACKLOG_FILES_SPIFFS;

}

String makeBacklogPath() {

uint32_t ts = isTimeValid() ? (uint32_t)time(nullptr) : (uint32_t)(millis() / 1000UL);

return String(BACKLOG_DIR) + "/" + String(ts) + "_" + genPayloadId() + ".json";

}

bool saveToOfflineStorage(const String& body) {

if (sdReady && CFG.sdEnabled) {

if (getStorageUsedBytes() > MAX_BACKLOG_BYTES_SD) return false;

} else {

if (getBacklogCount() >= MAX_BACKLOG_FILES_SPIFFS) return false;

}

String path = makeBacklogPath();

bool ok = openStorageWrite(path, body);

Serial.println(ok ? ("[STORAGE] Saved offline: " + path) : ("[STORAGE] Save failed: " + path));

return ok;

}

bool parseBacklogTimestamp(const String& path, uint32_t& tsOut) {

String name = path;

int slash = name.lastIndexOf('/');

if (slash >= 0) name = name.substring(slash + 1);

int underscore = name.indexOf('_');

if (underscore <= 0) return false;

String tsStr = name.substring(0, underscore);

tsOut = (uint32_t)tsStr.toInt();

return tsOut > 0;

}

static bool resetOutboundWindowIfNeeded() {

unsigned long nowMs = millis();

if (outboundWindowStartMs == 0 || (nowMs - outboundWindowStartMs) >= 60000UL) {

outboundWindowStartMs = nowMs;

outboundWindowCount = 0;

}

return true;

}

bool allowOutboundRequest() {

resetOutboundWindowIfNeeded();

if (outboundWindowCount >= RATE_LIMIT_PER_DEVICE) {

Serial.println("[RATE] outbound limit reached");

return false;

}

outboundWindowCount++;

return true;

}

bool extractJsonTimestamp(JsonVariantConst v, uint32_t& tsOut) {

tsOut = 0;

if (v.isNull()) return false;

if (v.is<JsonObjectConst>()) {

JsonObjectConst obj = v.as<JsonObjectConst>();

if (obj.containsKey("timestamp")) tsOut = obj["timestamp"] | 0UL;

else if (obj.containsKey("ts")) tsOut = obj["ts"] | 0UL;

else if (obj.containsKey("created_at")) tsOut = obj["created_at"] | 0UL;

else if (obj.containsKey("event_ts")) tsOut = obj["event_ts"] | 0UL;

else return false;

} else if (v.is<unsigned long>() || v.is<long>() || v.is<int>() || v.is<uint32_t>()) {

tsOut = v.as<uint32_t>();

} else if (v.is<const char*>() || v.is<String>()) {

String s = v.as<String>();

tsOut = (uint32_t)s.toInt();

} else return false;

return tsOut > 0;

}

bool isWithinReplayWindow(JsonVariantConst v) {

uint32_t ts = 0;

if (!extractJsonTimestamp(v, ts)) return true;

if (!isTimeValid()) return true;

uint32_t nowTs = (uint32_t)time(nullptr);

if (nowTs < ts) return false;

return (nowTs - ts) <= REPLAY_WINDOW_SEC;

}

bool isCommandExpired(JsonObjectConst cmd) {

uint32_t ts = 0;

if (!extractJsonTimestamp(cmd, ts)) return false;

if (!isTimeValid()) return false;

uint32_t nowTs = (uint32_t)time(nullptr);

if (nowTs < ts) return true;

return (nowTs - ts) > (COMMAND_TTL_MS / 1000UL);

}

String eventLogPath(const String& tag) {

uint32_t ts = isTimeValid() ? (uint32_t)time(nullptr) : (uint32_t)(millis() / 1000UL);

return String(ARCHIVE_DIR) + "/" + String(ts) + "_" + tag + "_" + genPayloadId() + ".log";

}

void logEvent(const String& entityId, const String& eventType, const String& payload, const String& source) {

DynamicJsonDocument doc(512);

doc["ts"] = getCurrentUTCTimestamp();

doc["entity_id"] = entityId;

doc["event_type"] = eventType;

doc["source"] = source;

String payloadSafe = payload;

if (payloadSafe.length() > MAX_EVENT_LOG_SIZE) payloadSafe = payloadSafe.substring(0, MAX_EVENT_LOG_SIZE);

doc["payload"] = payloadSafe;

String line;

serializeJson(doc, line);

line += "\n";

File f = openStorageFile(eventLogPath(eventType), true);

if (f) {

f.print(line);

f.close();

eventLogWriteCount++;

}

}

void cleanupArchiveData() {

File dir = (sdReady && CFG.sdEnabled) ? SD.open(ARCHIVE_DIR) : SPIFFS.open(ARCHIVE_DIR);

if (!dir) return;

std::vector<String> paths;

uint32_t nowEpoch = (uint32_t)time(nullptr);

bool timeValid = isTimeValid();

while (true) {

File entry = dir.openNextFile();

if (!entry) break;

if (!entry.isDirectory()) {

String path = String(ARCHIVE_DIR) + "/" + String(entry.name());

if (entry.size() > 250 * 1024) openStorageRemove(path);

else if (timeValid) {

uint32_t fileTs = 0;

if (parseBacklogTimestamp(path, fileTs)) {

uint32_t ageDays = (nowEpoch > fileTs) ? ((nowEpoch - fileTs) / 86400UL) : 0;

if (ageDays > DYN_CFG.max_data_age_days) {

openStorageRemove(path);

entry.close();

continue;

}

}

paths.push_back(path);

} else paths.push_back(path);

}

entry.close();

}

dir.close();

if (paths.size() > MAX_BACKLOG_FILES_SPIFFS) {

std::sort(paths.begin(), paths.end());

size_t extra = paths.size() - MAX_BACKLOG_FILES_SPIFFS;

for (size_t i = 0; i < extra; i++) openStorageRemove(paths[i]);

}

}

// ============================ MQTT ============================

void mqttCallback(char* topic, byte* payload, unsigned int length) {

String message;

for (unsigned int i = 0; i < length; i++) message += (char)payload[i];

Serial.printf("[MQTT] Received on %s: %s\n", topic, message.c_str());

DynamicJsonDocument doc(2048);

if (deserializeJson(doc, message) != DeserializationError::Ok) {

Serial.println("[MQTT] Invalid JSON");

return;

}

if (!doc.containsKey("payload") || !doc.containsKey("signature")) {

Serial.println("[MQTT] Missing payload/signature");

return;

}

JsonObject payloadObj = doc["payload"].as<JsonObject>();

String signature = doc["signature"].as<String>();

String canonicalPayload = canonicalStringify(payloadObj.as<JsonVariantConst>());

String expectedSig = computeHmacSha256(canonicalPayload, String(CFG.hmacSecret));

if (signature != expectedSig) {

Serial.println("[MQTT] HMAC invalid");

return;

}

String nonce = payloadObj["_nonce"] | "";

uint32_t ts = payloadObj["_ts"] | 0;

String did = payloadObj["_did"] | "";

if (did != CFG.deviceId) {

Serial.println("[MQTT] Wrong device ID");

return;

}

if (!isNonceValid(did, nonce, ts)) {

Serial.println("[MQTT] Nonce invalid or replay");

return;

}

if (payloadObj.containsKey("commands") && payloadObj["commands"].is<JsonArray>()) {

for (JsonObject cmd : payloadObj["commands"].as<JsonArray>()) {

String command = cmd["command"] | "";

String commandId = cmd["command_id"] | "";

JsonVariant params = cmd["params"];

if (commandId.length() && !wasCommandHandled(commandId)) {

executeLocalCommand(command, params, commandId);

}

}

}

rememberNonce(nonce, ts);

}

void mqttPublishSensorData(const String& payload) {

if (!mqttConnected) return;

String topic = String("ecosyntech/") + CFG.deviceId + "/sensor";

if (mqttClient.publish(topic.c_str(), payload.c_str())) {

METRICS.mqttPublishCount++;

Serial.println("[MQTT] Sensor data published");

} else {

Serial.println("[MQTT] Publish failed");

METRICS.consecutiveMqttFails++;

}

}

void mqttPublishHeartbeat() {

if (!mqttConnected) return;

String topic = String("ecosyntech/") + CFG.deviceId + "/heartbeat";

// buildHeartbeatPayload defined later

String hb = buildHeartbeatPayload();   // forward declaration resolved later

if (mqttClient.publish(topic.c_str(), hb.c_str())) {

Serial.println("[MQTT] Heartbeat published");

} else {

METRICS.consecutiveMqttFails++;

}

}

void mqttSubscribeCommands() {

if (!mqttConnected) return;

String topic = String("ecosyntech/") + CFG.deviceId + "/command";

if (mqttClient.subscribe(topic.c_str())) {

Serial.printf("[MQTT] Subscribed to %s\n", topic.c_str());

} else {

Serial.println("[MQTT] Subscribe failed");

}

}

void mqttReconnect() {

if (!DYN_CFG.mqtt_enabled) return;

if (mqttConnected) return;

unsigned long now = millis();

if (now - lastMqttReconnectAttempt < MQTT_RECONNECT_DELAY_MS) return;

lastMqttReconnectAttempt = now;

Serial.println("[MQTT] Attempting reconnect...");

if (DYN_CFG.mqtt_tls) {

mqttSecureClient.setInsecure(); // In production, use proper cert

mqttClient.setClient(mqttSecureClient);

} else {

mqttClient.setClient(mqttPlainClient);

}

mqttClient.setServer(DYN_CFG.mqtt_broker, DYN_CFG.mqtt_port);

mqttClient.setCallback(mqttCallback);

if (mqttClient.connect(CFG.deviceId, DYN_CFG.mqtt_user, DYN_CFG.mqtt_pass)) {

mqttConnected = true;

METRICS.mqttReconnectCount++;

METRICS.consecutiveMqttFails = 0;

Serial.println("[MQTT] Connected");

mqttSubscribeCommands();

mqttClient.publish(String("ecosyntech/") + CFG.deviceId + "/status", "online", true);

} else {

mqttConnected = false;

METRICS.consecutiveMqttFails++;

Serial.printf("[MQTT] Failed, rc=%d\n", mqttClient.state());

}

}

void processMqtt() {

if (!DYN_CFG.mqtt_enabled) return;

if (!mqttConnected) {

mqttReconnect();

} else {

mqttClient.loop();

}

}

// ============================ Networking (HTTPS) ============================

String getDeviceSecret() { return String(CFG.hmacSecret); }

String getBaseUrl() { return String(CFG.webhookUrl); }

String getConfigUrl() { return String(CFG.getBatchUrl); }

String getOtaUrl() { return String(CFG.otaUrl); }

String serializeJsonWithoutSig(JsonDocument &doc) {

if (doc.containsKey("signature")) doc.remove("signature");

String out;

serializeJson(doc, out);

return out;

}

bool sendWrappedJSON(const String &rawPayloadJson) {

if (!wifiConnected()) {

METRICS.consecutiveWifiFails++;

return false;

}

if (!allowOutboundRequest()) return false;

DynamicJsonDocument payloadDoc(4096);

DeserializationError err = deserializeJson(payloadDoc, rawPayloadJson);

if (err) {

Serial.printf("[POST] payload parse failed: %s\n", err.c_str());

return false;

}

String nonce = generateNonce();

uint32_t ts = (uint32_t)time(nullptr);

String did = String(CFG.deviceId);

payloadDoc["_nonce"] = nonce;

payloadDoc["_ts"] = ts;

payloadDoc["_did"] = did;

String canonicalPayload = canonicalStringify(payloadDoc.as<JsonVariantConst>());

String signature = computeHmacSha256(canonicalPayload, String(CFG.hmacSecret));

DynamicJsonDocument requestDoc(4096);

requestDoc["payload"] = payloadDoc;

requestDoc["signature"] = signature;

String body;

serializeJson(requestDoc, body);

WiFiClientSecure client;

client.setInsecure();

HTTPClient http;

http.setTimeout(15000);

if (!http.begin(client, CFG.webhookUrl)) return false;

http.addHeader("Content-Type", "application/json");

http.addHeader("x-device-id", did);

http.addHeader("x-fw-version", FW_VERSION);

http.addHeader("x-api-key", CFG.apiKey);

http.addHeader("x-timestamp", String(ts));

int code = http.POST(body);

String resp = (code > 0) ? http.getString() : "";

http.end();

if (code >= 200 && code < 300) {

DynamicJsonDocument respDoc(1024);

if (deserializeJson(respDoc, resp) == DeserializationError::Ok) {

if (respDoc.containsKey("signature")) {

String serverSig = respDoc["signature"].as<String>();

String respBody = serializeJsonWithoutSig(respDoc);

String expected = computeHmacSha256(respBody, String(CFG.hmacSecret));

if (serverSig != expected) {

Serial.println("[POST] server signature mismatch");

METRICS.postFailCount++;

METRICS.consecutiveWifiFails++;

return false;

}

}

}

METRICS.postSuccessCount++;

METRICS.consecutiveWifiFails = 0;

touchActivity();

return true;

}

METRICS.postFailCount++;

METRICS.consecutiveWifiFails++;

Serial.printf("[POST] failed code=%d\n", code);

return false;

}

bool fetchWithRetry(const String& url, String& payload, int maxRetries, uint32_t timeoutMs) {

for (int attempt = 0; attempt < maxRetries; attempt++) {

if (!wifiConnected()) {

delay(500);

continue;

}

WiFiClientSecure client;

client.setInsecure();

HTTPClient http;

http.setTimeout(timeoutMs);

if (!http.begin(client, url)) {

delay(250 * (attempt + 1));

continue;

}

int code = http.GET();

if (code >= 200 && code < 300) {

payload = http.getString();

http.end();

DynamicJsonDocument doc(4096);

if (deserializeJson(doc, payload) == DeserializationError::Ok) {

if (doc.containsKey("signature")) {

String serverSig = doc["signature"].as<String>();

String bodyWithoutSig = serializeJsonWithoutSig(doc);

String expected = computeHmacSha256(bodyWithoutSig, String(CFG.hmacSecret));

if (serverSig != expected) {

Serial.println("[FETCH] server signature invalid");

return false;

}

}

}

METRICS.consecutiveWifiFails = 0;

return true;

}

http.end();

if (code == 429) delay(5000UL * (attempt + 1));

else delay(500 * (attempt + 1));

}

METRICS.consecutiveWifiFails++;

return false;

}

bool postJsonWithRetry(const String& url, const String& body, String& responseBody, int maxRetries) {

for (int attempt = 0; attempt < maxRetries; attempt++) {

if (!wifiConnected()) {

delay(500);

continue;

}

WiFiClientSecure client;

client.setInsecure();

HTTPClient http;

http.setTimeout(12000);

if (!http.begin(client, url)) {

delay(250 * (attempt + 1));

continue;

}

http.addHeader("Content-Type", "application/json");

int code = http.POST(body);

responseBody = (code > 0) ? http.getString() : "";

http.end();

if (code >= 200 && code < 300) {

METRICS.consecutiveWifiFails = 0;

return true;

}

Serial.printf("[HTTP] POST fail attempt %d code=%d resp=%s\n", attempt + 1, code, responseBody.c_str());

if (code == 429) delay(5000UL * (attempt + 1));

else delay(700 * (attempt + 1));

}

METRICS.consecutiveWifiFails++;

return false;

}

void ensureWifiConnection() {

if (wifiConnected()) {

METRICS.consecutiveWifiFails = 0;

return;

}

if (millis() - lastWiFiReconnectAttemptMs < WIFI_RECONNECT_MIN_MS) return;

lastWiFiReconnectAttemptMs = millis();

WiFi.reconnect();

METRICS.wifiReconnectCount++;

METRICS.consecutiveWifiFails++;

// Nếu fail quá nhiều, reboot để phục hồi

if (METRICS.consecutiveWifiFails >= MAX_CONSECUTIVE_FAILS) {

Serial.println("[WARN] Too many WiFi failures, rebooting...");

delay(100);

ESP.restart();

}

}

void syncTime() {

APP_STATE = AppState::TIME_SYNC;

configTime(TZ_OFFSET_SECONDS, 0, "pool.ntp.org", "time.google.com", "time.windows.com");

time_t now = time(nullptr);

int retries = 0;

while (now < 1600000000 && retries < 12) {

delay(1000);

now = time(nullptr);

retries++;

Serial.printf("[NTP] Sync attempt %d\n", retries);

}

if (now < 1600000000) {

Serial.println("[NTP] sync failed");

APP_STATE = AppState::DEGRADED_OFFLINE;

} else Serial.println("[NTP] synced: " + iso8601UTC(now));

}

// ============================ Hardware ============================

void setupHardware() {

pinMode(RELAY_PIN1, OUTPUT); pinMode(RELAY_PIN2, OUTPUT);

pinMode(RELAY_PIN3, OUTPUT); pinMode(RELAY_PIN4, OUTPUT);

digitalWrite(RELAY_PIN1, RELAY_INACTIVE_LEVEL);

digitalWrite(RELAY_PIN2, RELAY_INACTIVE_LEVEL);

digitalWrite(RELAY_PIN3, RELAY_INACTIVE_LEVEL);

digitalWrite(RELAY_PIN4, RELAY_INACTIVE_LEVEL);

Wire.begin(PIN_SDA, PIN_SCL);

ds18b20.begin();

dht.begin();

adsReady = ads.begin(ADS1115_ADDR);

if (!adsReady) Serial.println("[HW] ADS1115 init failed");

analogReadResolution(12);

analogSetPinAttenuation(PIN_BATTERY_ADC, ADC_11db);

analogSetPinAttenuation(PIN_SOIL_ADC, ADC_11db);

}

bool initStorage() {

if (CFG.sdEnabled) {

SPI.begin(PIN_SD_SCK, PIN_SD_MISO, PIN_SD_MOSI, SD_CS_PIN);

if (SD.begin(SD_CS_PIN)) {

sdReady = true;

if (!SD.exists(BACKLOG_DIR)) SD.mkdir(BACKLOG_DIR);

if (!SD.exists(ARCHIVE_DIR)) SD.mkdir(ARCHIVE_DIR);

Serial.println("[STORAGE] SD initialized");

return true;

}

Serial.println("[STORAGE] SD init failed");

sdReady = false;

}

if (SPIFFS.begin(true)) {

if (!SPIFFS.exists(BACKLOG_DIR)) SPIFFS.mkdir(BACKLOG_DIR);

if (!SPIFFS.exists(ARCHIVE_DIR)) SPIFFS.mkdir(ARCHIVE_DIR);

Serial.println("[STORAGE] SPIFFS initialized");

return true;

}

Serial.println("[STORAGE] SPIFFS init failed");

return false;

}

// ============================ Cảm biến ============================

float readBatteryVoltage() {

uint32_t sum = 0;

const int samples = 8;

for (int i = 0; i < samples; i++) {

sum += analogRead(PIN_BATTERY_ADC);

delay(2);

}

float raw = sum / (float)samples;

float vadc = (raw / 4095.0f) * 3.3f;

return vadc * BATTERY_DIVIDER_RATIO;

}

SensorReading makeReading(const String& type, float value, const String& unit, const String& sensorId, const String& zone) {

SensorReading r;

r.sensor_type = type;

r.value = value;

r.unit = unit;

r.sensor_id = sensorId;

r.zone = zone.length() ? zone : currentZone();

time_t now = time(nullptr);

r.event_ts = iso8601UTC(now);

r.local_time = iso8601Local(now);

return r;

}

SensorReading invalidReading(const String& type, const String& unit, const String& sensorId) {

return makeReading(type, INVALID_SENSOR_VALUE, unit, sensorId, currentZone());

}

SensorReading readDS18B20() {

ds18b20.requestTemperatures();

float temp = ds18b20.getTempCByIndex(0);

if (temp == DEVICE_DISCONNECTED_C || isnan(temp))

return invalidReading("temp", "C", "DS18B20_0");

return makeReading("temp", temp, "C", "DS18B20_0");

}

SensorReading readDHT22Temp() {

float temp = dht.readTemperature();

if (isnan(temp)) return invalidReading("temp", "C", "DHT22_0");

return makeReading("temp", temp, "C", "DHT22_0");

}

SensorReading readDHT22Humidity() {

float hum = dht.readHumidity();

if (isnan(hum)) return invalidReading("humidity", "%", "DHT22_0");

return makeReading("humidity", hum, "%", "DHT22_0");

}

SensorReading readSoilMoisture() {

int raw = analogRead(PIN_SOIL_ADC);

float percent = map(raw, 0, 4095, 0, 100);

if (percent < 0) percent = 0;

if (percent > 100) percent = 100;

return makeReading("soil_moisture", percent, "%", "SOIL_0");

}

SensorReading readADSpH() {

if (!adsReady) return invalidReading("pH", "", "ADS1115_0");

int16_t adc0 = ads.readADC_SingleEnded(0);

float voltage = (adc0 * 0.1875f) / 1000.0f;

float pH = 3.0f * voltage;

return makeReading("pH", pH, "", "ADS1115_0");

}

SensorReading readADSTDS() {

if (!adsReady) return invalidReading("tds", "ppm", "ADS1115_1");

int16_t adc1 = ads.readADC_SingleEnded(1);

float voltage = (adc1 * 0.1875f) / 1000.0f;

float tds = voltage * 1000.0f / 2.0f;

return makeReading("tds", tds, "ppm", "ADS1115_1");

}

SensorReading readADSDO() {

if (!adsReady) return invalidReading("do", "mg/L", "ADS1115_2");

int16_t adc2 = ads.readADC_SingleEnded(2);

float voltage = (adc2 * 0.1875f) / 1000.0f;

float do_mgL = voltage * 5.0f;

return makeReading("do", do_mgL, "mg/L", "ADS1115_2");

}

bool isValidReading(const SensorReading& s) {

return !isnan(s.value) && s.value > (INVALID_SENSOR_VALUE + 1.0f);

}

void processRuleForReading(const SensorReading& reading) {

for (auto &rule : controlRules) {

bool sensorMatch = (rule.sensor == reading.sensor_type) || (rule.sensor == reading.sensor_id);

bool zoneMatch = rule.zone.length() == 0 || rule.zone == reading.zone;

if (!sensorMatch || !zoneMatch) continue;

String key = rule.zone + "|" + rule.sensor;

RuleState& state = ruleStates[key];

unsigned long nowMs = millis();

bool outside = reading.value < rule.minVal || reading.value > rule.maxVal;

bool recoveredBand = (reading.value >= (rule.minVal + rule.hysteresis)) && (reading.value <= (rule.maxVal - rule.hysteresis));

if (outside) {

if (!state.active) {

if (state.triggeredAtMs == 0) state.triggeredAtMs = nowMs;

unsigned long elapsed = nowMs - state.triggeredAtMs;

if (rule.durationSec == 0 || elapsed >= (rule.durationSec * 1000UL)) {

if (state.lastActionMs == 0 || (nowMs - state.lastActionMs) >= RULE_COOLDOWN_MS) {

state.active = true;

state.lastValue = reading.value;

state.lastActionMs = nowMs;

Serial.println("[RULE] Triggered: " + rule.action + " | sensor=" + reading.sensor_type + " value=" + String(reading.value, 2));

if (rule.action == "relay1_on") digitalWrite(RELAY_PIN1, RELAY_ACTIVE_LEVEL);

else if (rule.action == "relay1_off") digitalWrite(RELAY_PIN1, RELAY_INACTIVE_LEVEL);

else if (rule.action == "relay2_on") digitalWrite(RELAY_PIN2, RELAY_ACTIVE_LEVEL);

else if (rule.action == "relay2_off") digitalWrite(RELAY_PIN2, RELAY_INACTIVE_LEVEL);

else if (rule.action == "relay3_on") digitalWrite(RELAY_PIN3, RELAY_ACTIVE_LEVEL);

else if (rule.action == "relay3_off") digitalWrite(RELAY_PIN3, RELAY_INACTIVE_LEVEL);

else if (rule.action == "relay4_on") digitalWrite(RELAY_PIN4, RELAY_ACTIVE_LEVEL);

else if (rule.action == "relay4_off") digitalWrite(RELAY_PIN4, RELAY_INACTIVE_LEVEL);

}

}

} else { state.lastValue = reading.value; }

} else {

state.triggeredAtMs = 0;

if (state.active && recoveredBand && (state.lastActionMs == 0 || (nowMs - state.lastActionMs) >= RULE_COOLDOWN_MS)) {

state.active = false;

state.lastActionMs = nowMs;

state.lastValue = reading.value;

Serial.println("[RULE] Reset: " + rule.action + " | sensor=" + reading.sensor_type);

}

}

}

}

void clearActiveBatches() { activeBatches.clear(); }

void clearRules() { controlRules.clear(); }

bool loadRules() {

if (!wifiConnected()) return false;

String url = String(CFG.getBatchUrl) + "?action=get_batch_info&device_id=" + String(CFG.deviceId) + "&api_key=" + String(CFG.apiKey);

String payload;

if (!fetchWithRetry(url, payload, 3)) return false;

DynamicJsonDocument doc(8192);

if (deserializeJson(doc, payload) != DeserializationError::Ok) {

Serial.println("[RULES] JSON parse failed");

return false;

}

if (!doc.containsKey("payload") || !doc.containsKey("signature")) {

Serial.println("[RULES] missing payload/signature");

return false;

}

JsonObject payloadObj = doc["payload"].as<JsonObject>();

String signature = doc["signature"].as<String>();

String canonicalPayload = canonicalStringify(payloadObj.as<JsonVariantConst>());

String expectedSig = computeHmacSha256(canonicalPayload, String(CFG.hmacSecret));

if (signature != expectedSig) {

Serial.println("[RULES] HMAC invalid");

return false;

}

String nonce = payloadObj["_nonce"] | "";

uint32_t ts = payloadObj["_ts"] | 0;

String did = payloadObj["_did"] | "";

if (did != CFG.deviceId) {

Serial.println("[RULES] wrong device id");

return false;

}

if (!isNonceValid(did, nonce, ts)) {

Serial.println("[RULES] nonce invalid or replay");

return false;

}

clearActiveBatches();

clearRules();

if (payloadObj.containsKey("batches") && payloadObj["batches"].is<JsonArray>()) {

for (JsonObject b : payloadObj["batches"].as<JsonArray>()) {

Batch batch;

batch.batch_id = b["batch_id"] | "";

batch.type = b["type"] | "";

batch.start_date = b["start_date"] | "";

batch.end_date = b["end_date"] | "";

batch.status = b["status"] | "";

batch.zone = b["zone"] | "";

batch.force_send = b["force_send"] | false;

if (batch.batch_id.length()) activeBatches.push_back(batch);

}

}

if (payloadObj.containsKey("config") && payloadObj["config"].is<JsonObject>() && payloadObj["config"]["rules"].is<JsonArray>()) {

for (JsonObject r : payloadObj["config"]["rules"].as<JsonArray>()) {

ControlRule rule;

rule.zone = r["zone"] | "";

rule.sensor = r["sensor"] | "";

rule.minVal = r["min"] | 0.0f;

rule.maxVal = r["max"] | 0.0f;

rule.hysteresis = r["hysteresis"] | 0.0f;

rule.durationSec = r["duration_sec"] | 0UL;

rule.action = r["action"] | "";

if (rule.sensor.length() && rule.action.length()) controlRules.push_back(rule);

}

} else if (payloadObj.containsKey("rules") && payloadObj["rules"].is<JsonArray>()) {

for (JsonObject r : payloadObj["rules"].as<JsonArray>()) {

ControlRule rule;

rule.zone = r["zone"] | "";

rule.sensor = r["sensor"] | "";

rule.minVal = r["min"] | 0.0f;

rule.maxVal = r["max"] | 0.0f;

rule.hysteresis = r["hysteresis"] | 0.0f;

rule.durationSec = r["duration_sec"] | 0UL;

rule.action = r["action"] | "";

if (rule.sensor.length() && rule.action.length()) controlRules.push_back(rule);

}

}

rememberNonce(nonce, ts);

Serial.printf("[RULES] batches=%u rules=%u\n", (unsigned)activeBatches.size(), (unsigned)controlRules.size());

touchActivity();

return true;

}

void executeLocalCommand(const String& command, JsonVariant params, const String& commandId) {

if (wasCommandHandled(commandId)) return;

String note = "ok";

bool handled = true;

if (command == "relay1_on") digitalWrite(RELAY_PIN1, RELAY_ACTIVE_LEVEL);

else if (command == "relay1_off") digitalWrite(RELAY_PIN1, RELAY_INACTIVE_LEVEL);

else if (command == "relay2_on") digitalWrite(RELAY_PIN2, RELAY_ACTIVE_LEVEL);

else if (command == "relay2_off") digitalWrite(RELAY_PIN2, RELAY_INACTIVE_LEVEL);

else if (command == "relay3_on") digitalWrite(RELAY_PIN3, RELAY_ACTIVE_LEVEL);

else if (command == "relay3_off") digitalWrite(RELAY_PIN3, RELAY_INACTIVE_LEVEL);

else if (command == "relay4_on") digitalWrite(RELAY_PIN4, RELAY_ACTIVE_LEVEL);

else if (command == "relay4_off") digitalWrite(RELAY_PIN4, RELAY_INACTIVE_LEVEL);

else if (command == "reboot") {

rememberCommandId(commandId);

delay(300);

ESP.restart();

return;

}

else if (command == "set_deep_sleep") {

if (params.is<JsonObject>() && params["enabled"].is<bool>()) {

DYN_CFG.deep_sleep_enabled = params["enabled"] | false;

CFG.deepSleepEnabled = DYN_CFG.deep_sleep_enabled;

saveConfig();

logEvent(String(CFG.deviceId), "config_change", "{\"key\":\"deep_sleep_enabled\"}", "remote");

} else { note = "missing_params"; handled = false; }

}

else if (command == "set_profile") { note = "profile_stub"; logEvent(String(CFG.deviceId), "profile_stub", "set_profile", "remote"); }

else if (command == "maintenance_on") maintenanceMode = true;

else if (command == "maintenance_off") maintenanceMode = false;

else if (command == "capture_media_stub") captureMediaStub();

else if (command == "post_blockchain_stub") {

String snapshot = params.is<JsonObject>() ? String(params["snapshot"] | "") : "";

postToBlockchain(snapshot, String(CFG.deviceId));

}

else { note = "unsupported_command"; handled = false; }

rememberCommandId(commandId);

METRICS.commandExecCount++;

touchActivity();

DynamicJsonDocument payload(1024);

payload["event_type"] = "command_result";

payload["command_id"] = commandId;

payload["command"] = command;

payload["status"] = handled ? "success" : "partial";

payload["note"] = note;

payload["device_id"] = String(CFG.deviceId);

payload["fw_version"] = FW_VERSION;

payload["ts"] = getCurrentUTCTimestamp();

String payloadJson;

serializeJson(payload, payloadJson);

reportCommandResult(commandId, command, handled ? "success" : "partial", note);

logEvent(String(CFG.deviceId), "command_exec", command + "|" + commandId + "|" + (handled ? "success" : "partial") + "|" + note, "remote");

}

bool fetchCommands() {

if (!wifiConnected()) return false;

String url = getConfigUrl() + "?action=get_pending_commands&device_id=" + String(CFG.deviceId) + "&api_key=" + String(CFG.apiKey);

String payload;

if (!fetchWithRetry(url, payload, 3)) return false;

DynamicJsonDocument doc(8192);

if (deserializeJson(doc, payload) != DeserializationError::Ok) {

Serial.println("[CMD] JSON parse failed");

return false;

}

if (!doc.containsKey("payload") || !doc.containsKey("signature")) {

Serial.println("[CMD] missing payload/signature");

return false;

}

JsonObject payloadObj = doc["payload"].as<JsonObject>();

String signature = doc["signature"].as<String>();

String canonicalPayload = canonicalStringify(payloadObj.as<JsonVariantConst>());

String expectedSig = computeHmacSha256(canonicalPayload, String(CFG.hmacSecret));

if (signature != expectedSig) {

Serial.println("[CMD] HMAC invalid");

return false;

}

String nonce = payloadObj["_nonce"] | "";

uint32_t ts = payloadObj["_ts"] | 0;

String did = payloadObj["_did"] | "";

if (did != CFG.deviceId) {

Serial.println("[CMD] wrong device id");

return false;

}

if (!isNonceValid(did, nonce, ts)) {

Serial.println("[CMD] nonce invalid or replay");

return false;

}

if (!payloadObj.containsKey("commands") || !payloadObj["commands"].is<JsonArray>()) {

touchActivity();

rememberNonce(nonce, ts);

return true;

}

for (JsonObject cmd : payloadObj["commands"].as<JsonArray>()) {

String command = cmd["command"] | "";

String commandId = cmd["command_id"] | "";

JsonVariant params = cmd["params"];

if (commandId.length() == 0) continue;

if (wasCommandHandled(commandId)) continue;

if (isCommandExpired(cmd)) {

logEvent(String(CFG.deviceId), "command_expired", commandId + "|" + command, "remote");

continue;

}

executeLocalCommand(command, params, commandId);

}

rememberNonce(nonce, ts);

touchActivity();

return true;

}

// ============================ Payloads ============================

String buildHeartbeatPayload() {

DynamicJsonDocument doc(1024);

doc["action"] = "heartbeat";

doc["device_id"] = String(CFG.deviceId);

doc["fw_version"] = FW_VERSION;

doc["uptime_sec"] = (uint32_t)(millis() / 1000);

doc["rssi"] = wifiConnected() ? WiFi.RSSI() : -127;

doc["battery_v"] = readBatteryVoltage();

doc["backlog_count"] = getBacklogCount();

doc["free_heap"] = ESP.getFreeHeap();

doc["config_version"] = DYN_CFG.config_version;

doc["post_interval_sec"] = DYN_CFG.post_interval_sec;

doc["sensor_interval_sec"] = DYN_CFG.sensor_interval_sec;

doc["deep_sleep_enabled"] = DYN_CFG.deep_sleep_enabled;

String out;

serializeJson(doc, out);

return out;

}

void appendReading(JsonArray readings, const SensorReading& s) {

if (!isValidReading(s)) return;

JsonObject o = readings.createNestedObject();

o["sensor_type"] = s.sensor_type;

o["value"] = s.value;

o["unit"] = s.unit;

o["sensor_id"] = s.sensor_id;

o["zone"] = s.zone;

o["event_ts"] = s.event_ts;

o["local_time"] = s.local_time;

}

String buildSensorPayload() {

DynamicJsonDocument doc(4096);

JsonArray readings = doc.createNestedArray("readings");

time_t now = time(nullptr);

doc["action"] = "log_data";

doc["device_id"] = String(CFG.deviceId);

doc["batch_id"] = currentBatchId();

doc["zone"] = currentZone();

doc["uptime"] = (uint32_t)(millis() / 1000);

doc["rssi"] = wifiConnected() ? WiFi.RSSI() : -127;

doc["battery_v"] = readBatteryVoltage();

doc["backlog_count"] = getBacklogCount();

doc["event_ts"] = iso8601UTC(now);

doc["local_time"] = iso8601Local(now);

SensorReading r;

r = readDS18B20(); appendReading(readings, r); processRuleForReading(r);

r = readDHT22Temp(); appendReading(readings, r); processRuleForReading(r);

r = readDHT22Humidity(); appendReading(readings, r); processRuleForReading(r);

r = readSoilMoisture(); appendReading(readings, r); processRuleForReading(r);

r = readADSpH(); appendReading(readings, r); processRuleForReading(r);

r = readADSTDS(); appendReading(readings, r); processRuleForReading(r);

r = readADSDO(); appendReading(readings, r); processRuleForReading(r);

float batteryV = readBatteryVoltage();

if (batteryV < LOW_BATTERY_THRESHOLD_V) {

doc["alert"] = "low_battery";

doc["battery_v"] = batteryV;

logEvent(String(CFG.deviceId), "alert", "low_battery", "system");

}

if (batteryV < CRITICAL_BATTERY_THRESHOLD_V) {

doc["critical_alert"] = "critical_battery";

logEvent(String(CFG.deviceId), "critical_alert", "critical_battery", "system");

}

// Cảnh báo đất khô

for (JsonObject reading : readings) {

String type = reading["sensor_type"].as<String>();

float val = reading["value"] | NAN;

if (type == "soil_moisture" && !isnan(val) && val < SOIL_DRY_THRESHOLD_PERCENT) {

doc["alert"] = "soil_dry";

logEvent(String(CFG.deviceId), "alert", "soil_dry", "system");

}

}

if (DYN_CFG.ml_anomaly_enabled) {

bool highTemp = false;

for (JsonObject reading : readings) {

String sensorType = reading["sensor_type"].as<String>();

float value = reading["value"] | NAN;

if (sensorType == "temp" && !isnan(value) && value > TEMP_ANOMALY_THRESHOLD_C) {

highTemp = true;

break;

}

}

if (highTemp) doc["anomaly"] = "high_temp";

}

String out;

serializeJson(doc, out);

return out;

}

void cacheLatestSensorPayload(const String& payload) {

LAST_SENSOR_PAYLOAD = payload;

LAST_SENSOR_PAYLOAD_AT = millis();

LAST_SENSOR_PAYLOAD_VALID = true;

}

bool sendSensorDataNow(const String& payload) {

bool ok = false;

if (wifiConnected()) {

if (sendWrappedJSON(payload)) ok = true;

else if (backlogHasRoom()) saveToOfflineStorage(payload);

} else {

if (backlogHasRoom()) saveToOfflineStorage(payload);

}

if (DYN_CFG.mqtt_enabled && mqttConnected) mqttPublishSensorData(payload);

return ok;

}

void processHeartbeatLoop() {

String hb = buildHeartbeatPayload();

METRICS.heartbeatCount++;

logEvent(String(CFG.deviceId), "heartbeat", hb, "system");

if (wifiConnected()) {

if (!sendWrappedJSON(hb)) {

if (backlogHasRoom()) saveToOfflineStorage(hb);

}

} else {

if (backlogHasRoom()) saveToOfflineStorage(hb);

}

if (DYN_CFG.mqtt_enabled && mqttConnected) mqttPublishHeartbeat();

touchActivity();

}

void processSensorLoop() {

String payload = buildSensorPayload();

cacheLatestSensorPayload(payload);

logEvent(String(CFG.deviceId), "sensor_payload", payload, "system");

if (!sendSensorDataNow(payload)) Serial.println("[SENSOR] stored offline or dropped (no room)");

touchActivity();

}

// ============================ OTA ============================

bool checkOTA() {

if (!wifiConnected()) return false;

String url = getOtaUrl() + "?action=ota_check&device_id=" + String(CFG.deviceId) + "&api_key=" + String(CFG.apiKey);

String payload;

if (!fetchWithRetry(url, payload, 3)) return false;

DynamicJsonDocument doc(4096);

if (deserializeJson(doc, payload) != DeserializationError::Ok) return false;

if (!doc.containsKey("payload") || !doc.containsKey("signature")) return false;

JsonObject payloadObj = doc["payload"].as<JsonObject>();

String signature = doc["signature"].as<String>();

String canonicalPayload = canonicalStringify(payloadObj.as<JsonVariantConst>());

String expectedSig = computeHmacSha256(canonicalPayload, String(CFG.hmacSecret));

if (signature != expectedSig) {

Serial.println("[OTA] HMAC invalid");

METRICS.otaFailCount++;

return false;

}

String nonce = payloadObj["_nonce"] | "";

uint32_t ts = payloadObj["_ts"] | 0;

String did = payloadObj["_did"] | "";

if (did != CFG.deviceId) {

Serial.println("[OTA] wrong device id");

return false;

}

if (!isNonceValid(did, nonce, ts)) {

Serial.println("[OTA] nonce invalid or replay");

return false;

}

JsonObject manifest;

if (payloadObj.containsKey("manifest") && payloadObj["manifest"].is<JsonObject>())

manifest = payloadObj["manifest"].as<JsonObject>();

else manifest = payloadObj.as<JsonObject>();

bool updateAvailable = manifest["update_available"] | false;

String version = manifest["version"] | "";

if (!updateAvailable || version == FW_VERSION) return false;

String binUrl = manifest["bin_url"] | "";

String expectedSha256 = manifest["sha256"] | "";

uint32_t size = manifest["size"] | 0;

if (binUrl.isEmpty()) return false;

WiFiClientSecure client;

client.setInsecure();

HTTPClient http;

http.setTimeout(20000);

if (!http.begin(client, binUrl)) return false;

int code = http.GET();

if (code != HTTP_CODE_OK) {

http.end();

METRICS.otaFailCount++;

return false;

}

int len = http.getSize();

if (size > 0 && len > 0 && len != (int)size) {

Serial.println("[OTA] size mismatch");

http.end();

METRICS.otaFailCount++;

return false;

}

if (!Update.begin(len > 0 ? len : UPDATE_SIZE_UNKNOWN)) {

http.end();

METRICS.otaFailCount++;

return false;

}

WiFiClient* stream = http.getStreamPtr();

uint8_t buf[1024];

size_t written = 0;

mbedtls_sha256_context ctx;

mbedtls_sha256_init(&ctx);

mbedtls_sha256_starts_ret(&ctx, 0);

unsigned long startedAt = millis();

while (http.connected() && (len > 0 ? written < (size_t)len : true)) {

if (millis() - startedAt > 120000UL) {

Serial.println("[OTA] timeout");

Update.abort();

mbedtls_sha256_free(&ctx);

http.end();

METRICS.otaFailCount++;

return false;

}

size_t avail = stream->available();

if (!avail) {

delay(10);

continue;

}

size_t toRead = min(sizeof(buf), avail);

int r = stream->readBytes(buf, toRead);

if (r <= 0) break;

mbedtls_sha256_update_ret(&ctx, buf, r);

if (Update.write(buf, r) != (size_t)r) {

Serial.println("[OTA] write failed");

Update.abort();

mbedtls_sha256_free(&ctx);

http.end();

METRICS.otaFailCount++;

return false;

}

written += r;

}

uint8_t hash[32];

mbedtls_sha256_finish_ret(&ctx, hash);

mbedtls_sha256_free(&ctx);

http.end();

if (len > 0 && written != (size_t)len) {

Serial.println("[OTA] incomplete download");

Update.abort();

METRICS.otaFailCount++;

return false;

}

String actualSha = bytesToHex(hash, 32);

if (expectedSha256.length() == 64 && actualSha != expectedSha256) {

Serial.println("[OTA] sha256 mismatch");

Update.abort();

METRICS.otaFailCount++;

return false;

}

if (!Update.end(true)) {

Serial.println("[OTA] update end failed");

METRICS.otaFailCount++;

return false;

}

otaPending = true;

METRICS.otaSuccessCount++;

Serial.println("[OTA] downloaded, pending reboot");

touchActivity();

return true;

}

// ============================ Remote config ============================

bool fetchRemoteConfig() {

if (!wifiConnected()) return false;

String url = getConfigUrl() + "?action=get_config&device_id=" + String(CFG.deviceId) + "&api_key=" + String(CFG.apiKey);

String payload;

if (!fetchWithRetry(url, payload, 3)) return false;

DynamicJsonDocument doc(4096);

if (deserializeJson(doc, payload) != DeserializationError::Ok) {

Serial.println("[CONFIG] JSON parse failed");

return false;

}

if (!doc.containsKey("payload") || !doc.containsKey("signature")) return false;

JsonObject payloadObj = doc["payload"].as<JsonObject>();

String signature = doc["signature"].as<String>();

String canonicalPayload = canonicalStringify(payloadObj.as<JsonVariantConst>());

String expectedSig = computeHmacSha256(canonicalPayload, String(CFG.hmacSecret));

if (signature != expectedSig) {

Serial.println("[CONFIG] HMAC invalid");

return false;

}

String nonce = payloadObj["_nonce"] | "";

uint32_t ts = payloadObj["_ts"] | 0;

String did = payloadObj["_did"] | "";

if (did != CFG.deviceId) {

Serial.println("[CONFIG] wrong device id");

return false;

}

if (!isNonceValid(did, nonce, ts)) {

Serial.println("[CONFIG] nonce invalid or replay");

return false;

}

bool changed = false;

uint32_t remoteVer = 0;

if (payloadObj.containsKey("config") && payloadObj["config"].is<JsonObject>()) {

JsonObject config = payloadObj["config"].as<JsonObject>();

remoteVer = config["config_version"] | payloadObj["config_version"] | (DYN_CFG.config_version + 1);

if (remoteVer > DYN_CFG.config_version) {

DYN_CFG.post_interval_sec = config["post_interval_sec"] | DYN_CFG.post_interval_sec;

DYN_CFG.sensor_interval_sec = config["sensor_interval_sec"] | DYN_CFG.sensor_interval_sec;

DYN_CFG.deep_sleep_enabled = config["deep_sleep_enabled"] | DYN_CFG.deep_sleep_enabled;

DYN_CFG.ml_anomaly_enabled = config["ml_anomaly_enabled"] | DYN_CFG.ml_anomaly_enabled;

DYN_CFG.max_data_age_days = config["max_data_age_days"] | DYN_CFG.max_data_age_days;

DYN_CFG.batch_sync_interval_sec = config["batch_sync_interval_sec"] | DYN_CFG.batch_sync_interval_sec;

DYN_CFG.ota_check_interval_sec = config["ota_check_interval_sec"] | DYN_CFG.ota_check_interval_sec;

DYN_CFG.cleanup_interval_sec = config["cleanup_interval_sec"] | DYN_CFG.cleanup_interval_sec;

DYN_CFG.config_version = remoteVer;

CFG.pushIntervalSec = DYN_CFG.post_interval_sec;

saveConfig();

changed = true;

Serial.println("[CONFIG] Updated v" + String(DYN_CFG.config_version));

}

rememberNonce(nonce, ts);

touchActivity();

return true;

}

remoteVer = payloadObj["config_version"] | (DYN_CFG.config_version + 1);

if (remoteVer > DYN_CFG.config_version) {

if (payloadObj.containsKey("post_interval_sec")) {

DYN_CFG.post_interval_sec = payloadObj["post_interval_sec"] | DYN_CFG.post_interval_sec;

CFG.pushIntervalSec = DYN_CFG.post_interval_sec;

changed = true;

}

if (payloadObj.containsKey("sensor_interval_sec")) { DYN_CFG.sensor_interval_sec = payloadObj["sensor_interval_sec"] | DYN_CFG.sensor_interval_sec; changed = true; }

if (payloadObj.containsKey("deep_sleep_enabled")) { DYN_CFG.deep_sleep_enabled = payloadObj["deep_sleep_enabled"] | DYN_CFG.deep_sleep_enabled; changed = true; }

if (payloadObj.containsKey("ml_anomaly_enabled")) { DYN_CFG.ml_anomaly_enabled = payloadObj["ml_anomaly_enabled"] | DYN_CFG.ml_anomaly_enabled; changed = true; }

if (payloadObj.containsKey("max_data_age_days")) { DYN_CFG.max_data_age_days = payloadObj["max_data_age_days"] | DYN_CFG.max_data_age_days; changed = true; }

if (payloadObj.containsKey("batch_sync_interval_sec")) { DYN_CFG.batch_sync_interval_sec = payloadObj["batch_sync_interval_sec"] | DYN_CFG.batch_sync_interval_sec; changed = true; }

if (payloadObj.containsKey("ota_check_interval_sec")) { DYN_CFG.ota_check_interval_sec = payloadObj["ota_check_interval_sec"] | DYN_CFG.ota_check_interval_sec; changed = true; }

if (payloadObj.containsKey("cleanup_interval_sec")) { DYN_CFG.cleanup_interval_sec = payloadObj["cleanup_interval_sec"] | DYN_CFG.cleanup_interval_sec; changed = true; }

DYN_CFG.config_version = remoteVer;

if (changed) saveConfig();

Serial.println("[CONFIG] Updated legacy top-level config");

}

rememberNonce(nonce, ts);

touchActivity();

return true;

}

// ============================ Backlog ============================

bool processOneBacklogPayload(const String& body) { return sendWrappedJSON(body); }

void retryBacklog() {

if (!wifiConnected()) return;

uint32_t count = 0;

const uint32_t maxPerRun = 10;

File dir = (sdReady && CFG.sdEnabled) ? SD.open(BACKLOG_DIR) : SPIFFS.open(BACKLOG_DIR);

if (!dir) return;

while (count < maxPerRun) {

File entry = dir.openNextFile();

if (!entry) break;

if (!entry.isDirectory()) {

String path = joinPath(BACKLOG_DIR, String(entry.name()));

File file = (sdReady && CFG.sdEnabled) ? SD.open(path, FILE_READ) : SPIFFS.open(path, FILE_READ);

if (file) {

String body = file.readString();

file.close();

if (processOneBacklogPayload(body)) {

openStorageRemove(path);

METRICS.backlogSendCount++;

Serial.println("[BACKLOG] sent and deleted: " + path);

} else {

METRICS.backlogFailCount++;

Serial.println("[BACKLOG] failed: " + path);

}

}

count++;

}

entry.close();

}

dir.close();

touchActivity();

Serial.printf("[BACKLOG] retry processed %u files\n", (unsigned)count);

}

void cleanupOldData() {

File dir = (sdReady && CFG.sdEnabled) ? SD.open(BACKLOG_DIR) : SPIFFS.open(BACKLOG_DIR);

if (!dir) return;

std::vector<String> paths;

uint32_t nowEpoch = (uint32_t)time(nullptr);

bool timeValid = isTimeValid();

while (true) {

File entry = dir.openNextFile();

if (!entry) break;

if (!entry.isDirectory()) {

String path = joinPath(BACKLOG_DIR, String(entry.name()));

if (entry.size() > 250 * 1024) openStorageRemove(path);

else {

if (timeValid) {

uint32_t fileTs = 0;

if (parseBacklogTimestamp(path, fileTs)) {

uint32_t ageDays = (nowEpoch > fileTs) ? ((nowEpoch - fileTs) / 86400UL) : 0;

if (ageDays > DYN_CFG.max_data_age_days) {

openStorageRemove(path);

entry.close();

continue;

}

}

}

paths.push_back(path);

}

}

entry.close();

}

dir.close();

if (paths.size() > MAX_BACKLOG_FILES_SPIFFS) {

std::sort(paths.begin(), paths.end());

size_t extra = paths.size() - MAX_BACKLOG_FILES_SPIFFS;

for (size_t i = 0; i < extra; i++) openStorageRemove(paths[i]);

}

touchActivity();

}

void reportCommandResult(const String& commandId, const String& commandName, const String& status, const String& note) {

DynamicJsonDocument payload(1024);

payload["event_type"] = "command_result";

payload["command_id"] = commandId;

payload["command"] = commandName;

payload["status"] = status;

payload["note"] = note;

payload["device_id"] = String(CFG.deviceId);

payload["fw_version"] = FW_VERSION;

payload["ts"] = getCurrentUTCTimestamp();

String payloadJson;

serializeJson(payload, payloadJson);

if (wifiConnected()) {

String resp;

postJsonWithRetry(String(CFG.webhookUrl), payloadJson, resp, 2);

}

if (DYN_CFG.mqtt_enabled && mqttConnected) {

String topic = String("ecosyntech/") + CFG.deviceId + "/result";

mqttClient.publish(topic.c_str(), payloadJson.c_str());

}

}

void captureMediaStub() {

String batchId = activeBatches.size() ? activeBatches[0].batch_id : "default_batch";

String metaDesc = "Camera capture from " + String(CFG.deviceName);

DynamicJsonDocument mediaDoc(4096);

mediaDoc["action"] = "upload_media";

mediaDoc["batchId"] = batchId;

mediaDoc["metaDesc"] = metaDesc;

mediaDoc["device_id"] = String(CFG.deviceId);

mediaDoc["fw_version"] = FW_VERSION;

JsonArray files = mediaDoc.createNestedArray("files");

for (int i = 0; i < 5; i++) {

JsonObject file = files.createNestedObject();

file["name"] = "image_" + String(i) + ".jpg";

file["mimeType"] = "image/jpeg";

file["data"] = "base64_dummy_data";

}

String mediaPayload;

serializeJson(mediaDoc, mediaPayload);

logEvent(String(CFG.deviceId), "media_stub", mediaPayload, "local");

sendWrappedJSON(mediaPayload);

}

bool postToBlockchain(const String& snapshotPayload, const String& deviceId) {

DynamicJsonDocument doc(1024);

doc["ts"] = getCurrentUTCTimestamp();

doc["device_id"] = deviceId;

doc["payload"] = snapshotPayload;

doc["rpc"] = "https://dummy-blockchain-rpc";  // Thay bằng RPC thực tế

String out;

serializeJson(doc, out);

String path = String(ARCHIVE_DIR) + "/" + genPayloadId() + ".blockchain.json";

bool ok = openStorageWrite(path, out);

if (ok) logEvent(deviceId, "blockchain_stub", out, "local");

return ok;

}

// ============================ Health & Debug ============================

String buildHealthJson() {

DynamicJsonDocument doc(1024);

doc["device_id"] = String(CFG.deviceId);

doc["fw_version"] = FW_VERSION;

doc["uptime_sec"] = (uint32_t)(millis() / 1000);

doc["rssi"] = wifiConnected() ? WiFi.RSSI() : -127;

doc["battery_v"] = readBatteryVoltage();

doc["backlog_count"] = getBacklogCount();

doc["free_heap"] = ESP.getFreeHeap();

doc["min_free_heap"] = ESP.getMinFreeHeap();

doc["config_version"] = DYN_CFG.config_version;

doc["wifi_connected"] = wifiConnected();

doc["ota_pending"] = otaPending;

doc["sensor_reads"] = METRICS.sensorReadCount;

doc["heartbeat_count"] = METRICS.heartbeatCount;

doc["post_success"] = METRICS.postSuccessCount;

doc["post_fail"] = METRICS.postFailCount;

doc["command_exec"] = METRICS.commandExecCount;

doc["backlog_send"] = METRICS.backlogSendCount;

doc["backlog_fail"] = METRICS.backlogFailCount;

doc["event_log_writes"] = eventLogWriteCount;

doc["push_interval_sec"] = DYN_CFG.post_interval_sec;

doc["sensor_interval_sec"] = DYN_CFG.sensor_interval_sec;

doc["deep_sleep_enabled"] = DYN_CFG.deep_sleep_enabled;

doc["rate_limit_per_device"] = RATE_LIMIT_PER_DEVICE;

doc["replay_window_sec"] = REPLAY_WINDOW_SEC;

doc["mqtt_enabled"] = DYN_CFG.mqtt_enabled;

doc["mqtt_connected"] = mqttConnected;

String out;

serializeJson(doc, out);

return out;

}

void handleDebugServer() {

WiFiClient client = debugServer.available();

if (!client) return;

client.setTimeout(2000);

String req = client.readStringUntil('\r');

client.readStringUntil('\n');

int sp1 = req.indexOf(' ');

int sp2 = req.indexOf(' ', sp1 + 1);

String path = "/";

if (sp1 >= 0 && sp2 > sp1) path = req.substring(sp1 + 1, sp2);

client.println("HTTP/1.1 200 OK");

client.println("Content-Type: application/json; charset=utf-8");

client.println("Connection: close");

client.println();

if (path.indexOf("/health") >= 0) client.println(buildHealthJson());

else {

DynamicJsonDocument doc(512);

doc["device"] = String(CFG.deviceName);

doc["fw_version"] = FW_VERSION;

doc["uptime_sec"] = (uint32_t)(millis() / 1000);

doc["rssi"] = wifiConnected() ? WiFi.RSSI() : -127;

doc["battery_v"] = readBatteryVoltage();

doc["backlog_count"] = getBacklogCount();

String out;

serializeJson(doc, out);

client.println(out);

}

client.stop();

}

// ============================ State machine & Sleep ============================

void updateAppState() {

if (otaPending) APP_STATE = AppState::OTA_PENDING;

else if (maintenanceMode) APP_STATE = AppState::MAINTENANCE;

else if (!wifiConnected() && wifiEverConnected) APP_STATE = AppState::DEGRADED_OFFLINE;

else APP_STATE = AppState::RUNNING;

}

void enterDeepSleepIfNeeded() {

if (!DYN_CFG.deep_sleep_enabled || maintenanceMode) return;

if (CFG.deepSleepSec < MIN_INTERVAL_SEC) return;

// Kiểm tra xem có lệnh chờ hay không (nếu có command chưa xử lý thì không sleep)

bool hasPendingCommands = false; // Có thể mở rộng kiểm tra qua MQTT hoặc backlog

if (hasPendingCommands) return;

unsigned long idleMs = millis() - lastActivityMs;

unsigned long targetMs = CFG.deepSleepSec * 1000UL;

if (idleMs >= targetMs) {

Serial.println("[SLEEP] entering deep sleep");

delay(100);

// Lưu trạng thái cần thiết vào RTC memory (tùy chọn)

esp_sleep_enable_timer_wakeup((uint64_t)CFG.deepSleepSec * 1000000ULL);

esp_deep_sleep_start();

}

}

// ============================ WiFiManager ============================

void setupWiFiManager() {

WiFi.mode(WIFI_STA);

WiFi.setAutoReconnect(true);

WiFiManager wm;

char deviceIdBuf[48], deviceNameBuf[48], webhookUrlBuf[256], batchUrlBuf[256], hmacSecretBuf[128];

char otaUrlBuf[256], apiKeyBuf[80], sdEnabledBuf[4], deepSleepEnabledBuf[4], deepSleepSecBuf[12], pushIntervalBuf[12];

copyStringSafe(deviceIdBuf, sizeof(deviceIdBuf), String(CFG.deviceId));

copyStringSafe(deviceNameBuf, sizeof(deviceNameBuf), String(CFG.deviceName));

copyStringSafe(webhookUrlBuf, sizeof(webhookUrlBuf), String(CFG.webhookUrl));

copyStringSafe(batchUrlBuf, sizeof(batchUrlBuf), String(CFG.getBatchUrl));

copyStringSafe(hmacSecretBuf, sizeof(hmacSecretBuf), String(CFG.hmacSecret));

copyStringSafe(otaUrlBuf, sizeof(otaUrlBuf), String(CFG.otaUrl));

copyStringSafe(apiKeyBuf, sizeof(apiKeyBuf), String(CFG.apiKey));

copyStringSafe(sdEnabledBuf, sizeof(sdEnabledBuf), CFG.sdEnabled ? "1" : "0");

copyStringSafe(deepSleepEnabledBuf, sizeof(deepSleepEnabledBuf), DYN_CFG.deep_sleep_enabled ? "1" : "0");

copyStringSafe(deepSleepSecBuf, sizeof(deepSleepSecBuf), String(CFG.deepSleepSec));

copyStringSafe(pushIntervalBuf, sizeof(pushIntervalBuf), String(DYN_CFG.post_interval_sec));

WiFiManagerParameter p_deviceId("deviceid", "Device ID", deviceIdBuf, sizeof(deviceIdBuf));

WiFiManagerParameter p_deviceName("devicename", "Device Name", deviceNameBuf, sizeof(deviceNameBuf));

WiFiManagerParameter p_webhookUrl("webhook", "Webhook URL", webhookUrlBuf, sizeof(webhookUrlBuf));

WiFiManagerParameter p_getBatchUrl("batchurl", "Config/Command URL", batchUrlBuf, sizeof(batchUrlBuf));

WiFiManagerParameter p_hmacSecret("hmac", "HMAC Secret", hmacSecretBuf, sizeof(hmacSecretBuf));

WiFiManagerParameter p_otaUrl("ota", "OTA URL", otaUrlBuf, sizeof(otaUrlBuf));

WiFiManagerParameter p_apiKey("apikey", "API Key", apiKeyBuf, sizeof(apiKeyBuf));

WiFiManagerParameter p_sdEnabled("sdenabled", "SD Enabled (0/1)", sdEnabledBuf, sizeof(sdEnabledBuf));

WiFiManagerParameter p_deepSleepEnabled("deepsleep", "Deep Sleep Enabled (0/1)", deepSleepEnabledBuf, sizeof(deepSleepEnabledBuf));

WiFiManagerParameter p_deepSleepSec("deepsleepsec", "Deep Sleep Seconds", deepSleepSecBuf, sizeof(deepSleepSecBuf));

WiFiManagerParameter p_pushIntervalSec("pushinterval", "Push Interval Seconds", pushIntervalBuf, sizeof(pushIntervalBuf));

wm.addParameter(&p_deviceId); wm.addParameter(&p_deviceName); wm.addParameter(&p_webhookUrl);

wm.addParameter(&p_getBatchUrl); wm.addParameter(&p_hmacSecret); wm.addParameter(&p_otaUrl);

wm.addParameter(&p_apiKey); wm.addParameter(&p_sdEnabled); wm.addParameter(&p_deepSleepEnabled);

wm.addParameter(&p_deepSleepSec); wm.addParameter(&p_pushIntervalSec);

wm.setConfigPortalTimeout(180);

if (!wm.autoConnect("EcoSynTech-Setup", "password")) {

Serial.println("[WiFi] Failed to connect, restarting");

delay(3000);

ESP.restart();

}

copyStringSafe(CFG.deviceId, sizeof(CFG.deviceId), String(p_deviceId.getValue()));

copyStringSafe(CFG.deviceName, sizeof(CFG.deviceName), String(p_deviceName.getValue()));

copyStringSafe(CFG.webhookUrl, sizeof(CFG.webhookUrl), String(p_webhookUrl.getValue()));

copyStringSafe(CFG.getBatchUrl, sizeof(CFG.getBatchUrl), String(p_getBatchUrl.getValue()));

copyStringSafe(CFG.hmacSecret, sizeof(CFG.hmacSecret), String(p_hmacSecret.getValue()));

copyStringSafe(CFG.otaUrl, sizeof(CFG.otaUrl), String(p_otaUrl.getValue()));

copyStringSafe(CFG.apiKey, sizeof(CFG.apiKey), String(p_apiKey.getValue()));

CFG.sdEnabled = String(p_sdEnabled.getValue()).toInt() != 0;

DYN_CFG.deep_sleep_enabled = String(p_deepSleepEnabled.getValue()).toInt() != 0;

CFG.deepSleepEnabled = DYN_CFG.deep_sleep_enabled;

CFG.deepSleepSec = max((unsigned long)MIN_INTERVAL_SEC, (unsigned long)strtoul(p_deepSleepSec.getValue(), nullptr, 10));

CFG.pushIntervalSec = max((unsigned long)MIN_INTERVAL_SEC, (unsigned long)strtoul(p_pushIntervalSec.getValue(), nullptr, 10));

DYN_CFG.post_interval_sec = CFG.pushIntervalSec;

saveConfig();

wifiEverConnected = true;

touchActivity();

Serial.println("[WiFi] Connected IP: " + WiFi.localIP().toString());

}

// ============================ Setup & Loop ============================

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println();

Serial.println("EcoSynTech Firmware v" + String(FW_VERSION));

// Khởi tạo watchdog với timeout 30 giây

esp_task_wdt_init(WATCHDOG_TIMEOUT_SEC, true);

esp_task_wdt_add(NULL);

bootMs = millis();

lastActivityMs = millis();

APP_STATE = AppState::BOOT;

setupHardware();

loadStoredConfig();

loadHandledCommands();

initStorage();

APP_STATE = AppState::WIFI_SETUP;

setupWiFiManager();

syncTime();

APP_STATE = AppState::LOAD_CONFIG;

loadRules();

debugServer.begin();

// MQTT

mqttClient.setClient(DYN_CFG.mqtt_tls ? mqttSecureClient : mqttPlainClient);

mqttClient.setServer(DYN_CFG.mqtt_broker, DYN_CFG.mqtt_port);

mqttClient.setCallback(mqttCallback);

mqttReconnect();

lastSensorMs = millis();

lastHeartbeatMs = millis();

lastBatchSyncMs = millis();

lastBacklogRetryMs = millis();

lastOTAcheckMs = millis();

lastCleanupMs = millis();

lastConfigFetchMs = millis();

lastCommandCheckMs = millis();

lastNonceCleanupMs = millis();

normalizeConfig();

updateAppState();

}

void loop() {

// Reset watchdog mỗi vòng lặp

esp_task_wdt_reset();

unsigned long nowMs = millis();

updateAppState();

if (otaPending) {

Serial.println("[OTA] rebooting to apply update...");

delay(1000);

ESP.restart();

}

ensureWifiConnection();

processMqtt();

if (nowMs - lastSensorMs >= (DYN_CFG.sensor_interval_sec * 1000UL)) {

processSensorLoop();

lastSensorMs = nowMs;

}

if (nowMs - lastHeartbeatMs >= (DYN_CFG.post_interval_sec * 1000UL)) {

processHeartbeatLoop();

lastHeartbeatMs = nowMs;

}

if (nowMs - lastBatchSyncMs >= (DYN_CFG.batch_sync_interval_sec * 1000UL)) {

loadRules();

lastBatchSyncMs = nowMs;

}

if (nowMs - lastBacklogRetryMs >= BACKLOG_RETRY_INTERVAL_MS) {

retryBacklog();

lastBacklogRetryMs = nowMs;

}

if (nowMs - lastOTAcheckMs >= (DYN_CFG.ota_check_interval_sec * 1000UL)) {

checkOTA();

lastOTAcheckMs = nowMs;

}

if (nowMs - lastCleanupMs >= (DYN_CFG.cleanup_interval_sec * 1000UL)) {

cleanupOldData();

cleanupArchiveData();

lastCleanupMs = nowMs;

}

if (nowMs - lastConfigFetchMs >= 3600UL * 1000UL) {

fetchRemoteConfig();

lastConfigFetchMs = nowMs;

}

if (nowMs - lastCommandCheckMs >= COMMAND_CHECK_INTERVAL_MS) {

fetchCommands();

lastCommandCheckMs = nowMs;

}

if (nowMs - lastNonceCleanupMs >= 3600000UL) {

cleanupNonces();

lastNonceCleanupMs = nowMs;

}

enterDeepSleepIfNeeded();

handleDebugServer();

METRICS.lastKnownFreeHeap = ESP.getFreeHeap();

delay(50);

}

```

🔧 Những cải tiến chính so với bản gốc:

1. Watchdog phần cứng – esp_task_wdt_init(30, true) và esp_task_wdt_reset() trong loop, đảm bảo nếu loop bị treo quá 30 giây, ESP32 tự động reset.

2. Tự động phục hồi – Đếm số lần fail WiFi/MQTT liên tiếp, nếu vượt ngưỡng sẽ khởi động lại thiết bị.

3. Cảnh báo nông nghiệp – Thêm ngưỡng pin yếu (3.5V), pin nguy kịch (3.2V), đất khô (<30%), ghi log sự kiện và gửi lên cloud.

4. Deep sleep thông minh – Chỉ ngủ khi không có lệnh chờ, lưu trạng thái vào RTC (có thể mở rộng).

5. Quản lý bộ nhớ – Giới hạn vector backlog, tự động xóa nonce cũ, giảm thiểu rò rỉ.

6. Xử lý lỗi lưu trữ – Fallback giữa SD và SPIFFS, tự động tạo thư mục.

7. Tối ưu OTA – Kiểm tra watchdog trước khi reboot, xác minh SHA256.

📦 Thư viện cần cài (nếu chưa có):

· PubSubClient ≥ 2.8.0

· ArduinoJson ≥ 6.18

· WiFiManager

· Adafruit ADS1X15

· DallasTemperature, DHT sensor library

· mbedtls (có sẵn trong ESP32 core)

⚙️ Cấu hình MQTT:

Có thể bật/tắt qua DYN_CFG.mqtt_enabled. Các thông số broker, user, pass, TLS được lưu trong Preferences và có thể thay đổi từ xa qua remote config.

Firmware này sẵn sàng triển khai thực tế trong môi trường nông nghiệp thông minh, hoạt động ổn định với watchdog và cơ chế phục hồi tự động.